Control 2.0

To design a control circuit, the choice is between type 1,2 or 3. The main purpose of control circuit is to maintain the 8V output voltage, reduce output voltage ripple to <0.2V and the 100Hz ripple caused by the rectifier shown in Figure(--).

Even though type 1 control circuit is easy having only a single capacitor and resistor. It does not meet our requirement—0.2V output ripple when feeding alternating current as input whilst this project relies on the control circuit to remove it. Thus, having type 3 control circuit is the go-to choice. The transfer function for type 3 control circuit is given as:

Gcs=As1+sωck21+skωc2 #1

Where:

k is Venable’s k-factor

An integrator is inside the equation needed for the steady state accuracy. Two zeros are placed at a frequency below the crossover frequency (ωc) and two poles at a frequency above crossover frequency. The zeros provide a phase-lead, which helps reduce the phase lag of the control loop at the crossover frequency. The poles are there as we never realise a transfer function with more zero than poles which gives a phase lag again, but there are above the crossover frequency to minimize the effect on stability. As for now, this paper is discussing on implementation Venable’s design.

Crossover Frequency, ωc

Crossover frequency (ωc) is a value above the resonant frequency (ωresonant). It could be chosen between 2 till 10 as the multiplier. For this project, the team chose 4 as the multiplier due to making it too low won’t get enough loop gain at 100Hz and too high causes instability issue by high frequency noise which affect the control loop.  Thus, making the equation below.

ωc=4×ωresonant#2

While,

ωresonant=1LC#3

From the previous report, L = 49μH and C=940μF. Substitute into the equation we get:

ωc=449μ×940μ

ωc=18637.94

LC filter Transfer Function, Gfs

The LC filter transfer function (Gfs) is the mathematical baseline for the control circuit. Without it, the compensator in the control circuit would have to guess the output. This transfer function is also essential for finding the gain and phase boost for later. More importantly, this enables us to find the transfer function for the control circuit Gcs later.

To get the transfer function, consider the circuit below.

To get the transfer function,

Gfs=VoutsVXLFs

Gfs=RR+RC1+sRCCs2LC+sL+CRRCR+Rc +RR+Rc

For any sensible design and operating condition R≫Rc, the equation can be simplified as:

Gfs=1+sRcCs2LC+sLR+CRc+1

By substituting the L=49μ,Rc=0.1,C=940μ and R=5.

Gfs=1+s0.1940μs249μ940μ+s49μ5+940μ0.1+1

Gfs=1+9.4×10-5s4.61×10-8s2+1.038×10-4s+1

To get the phase margin and magnitude at crossover frequency (ωc)of the transfer function Gfs, Replace s with jωc.

To find magnitude,

Gfjωc=1+jωc9.4×10-5|4.61×10-8jωc2+1.038×10-4(jωc)+1|

Gfjωc=1+j1.752|-15.014+j1.935|

Gfjωc=0.1332

To find phase margin,

∠Gfjωc=tan-11.7521-tan-11.935-15.014+180°

The second part of the calculation needed to add another 180° due to the quadrant error. This is due to the -15.014+j1.935 sits in quadrant II while our calculator is limited to -90° to +90°. Thus, can’t directly input inside calculator.

∠Gfjωc=247.67° or-112.373°

Phase Boost, B and k-factor, k

The phase boost, B is required to know how much phase boost is needed for the control circuit. The desired phase margin (PM) is 60°. The equation is given by:

Phase Margin, PM=180-90+∠Gfjωc+B

B=PM-180+90-∠Gfjωc

B=60-180+90--112.373

B=82.373°

k-factor is needed for calculation. By using Venable’s formula,

k=tanB4+452

k=tan82.3734+452

k=4.856

A Value of amplifier circuit and pwm circuit, Kmisc

The value for Kmisc is essential as this is adding the pwm with the potential divider into the equation. The given equation is:

Kmisc=αKpwmVS0n

α is the potential divider value. Any value less than 5V can be chosen as the reference voltage since the max voltage accepted by the operational amplifier is 5V and supply voltage. Thus, this project chose 3V as the reference voltage. Giving the equations are:

α=VrefVout

α=38 or 0.375

Kpwm given equation is:

Kpwm=dmaxVb-Va

Where:

dmax is the maximum duty cycle output from the pwm chip.

Vb is the input threshold voltage for maximum duty cycle.

Va is the input threshold voltage for minimum duty cycle.

∴Kpwm=0.453.5-1

Kpwm=0.18

VS0 is the maximum input voltage—39.74V. This value is able to obtain from simulation or manually calculate which I wont fucking touch ahahahahahaaha.

n is the turn ratio of the transformer. Since the transformer being used is a 1 to 1 transformer, the n is equal to 1.

By substituting everything into the equation:

Kmisc=0.3750.1839.741

Kmisc=2.6824

Gain Required, G

The Gain required by the controller can be calculated with the following equation.

G=1KmiscGfjωc

This would provide us with some ideas and the essential to calculate the component value.

G=12.68240.1332

G=2.799

Integrator Gain, A

With the Gain obtained from the previous section, the integrator gain can get by this equation.

A=Gωck

A=2.79918637.944.856

A=10742.91

Transfer Function for controller, Gcs

With the obtained A value, the transfer function of the controller loop is possible to obtain with the equation below.

Gcs=As1+sωz11+sωz21+sωp11+sωp2

While

ωz1=ωz2=ωck=18637.944.856=8457.823

And

ωp1=ωp2=kωc=18637.944.856=41071.19

Giving

Gcs=10742.91s1+s8457.8231+s8457.8231+s41071.191+s41071.19

Gcs=1.812×1013s2+3.065×1017s+1.296×10217.153×103s3+5.876×1012s2+1.207×1017s

With the obtained Gcs, it is possible to plot the bode diagram which is shown in figure below.

Verifying the control

To verify our control circuit, the open loop transfer function graph and the closed loop transfer function graph need to be found.

The open loop Bode plot helps to prove the stability of the control system and the ability the reject the rectifier ripple. The open loop transfer function is given by:

Gloops=Gcs×Gfs×Kmisc

By substituting in the values, we got:

∴Gloops=4.563×109s3+1.257×1014s2+1.147×1018s+3.472×10213.295s5+2.781×105s4+6.239×109s3+1.840×1013s2+1.207×1017s

By plotting the graph using MATLab,

Figure (): the open loop phase and magnitude against frequency graph

As shown in the figure (), the gain margin is shown infinity. Which means the phase would not reach to -180° mark when the gain is above 0dB, which proves that the system is stable. Another thing to take note of is the phase margin, the graph shows that the phase margin is 60° at the crossover frequency which is the targeted phase margin. This ensures that the system is highly stable and won’t oscillate and capable of providing sufficient transient response when load changes.

The close loop Bode plot enables us to see the system response with reference to outside perspective. The given formula for close loop transfer function is:

Gclose-loops=Gloops1+Gloops

By substituting Gloops into Gclose-loops:

Gclose-loops=

1.50×1010s8+1.68×1015s7+6.72×1019s6+1.20×1024s5+1.10×1028s4+5.80×1031s3+2.02×1035s2+4.19×1038s10.86s10+1.83×106s9+1.34×1011s8+5.27×1015s7+1.17×1020s6+1.50×1024s5+1.28×1023s4+6.34×1031s3+2.17×1035s2+4.19×1038s

Having Gclose-loops, the plot margin can obtain as figure shown.

Figure (): The close loop bode plot with phase and magnitude

From figure (), it shows the magnitude at low frequencies is 0dB which satisfy our requirement. This shows that the system perfectly tracks the output voltage with 0 steady state error. Continue at the magnitude graph, it shows that the gain drops at frequency which is the desired output. The control system acts like a low-pass filter, which helps ignoring switching noise and doesn’t destabilize the feedback loop.

The calculation for components for Z1 and Z2

For a type 3 controller, there’s a Z1 and Z2 component which shown in figure ().

Figure (): the component for Z1                        Figure (): the component for Z2

To calculate components in Z1, the equation given to these are:

R3=R1k-1

C3=1R3ωck

Assume R1=10kΩ,

R3=10×1034.856-1

R3=2.593kΩ

C3=12.593×10318637.944.856

C3=9.389nF

To calculate components in Z2, the equation given to these are:

C2=1R1ωcG

C1=C2×k-1

R2=kωcC1

By substituting the obtained value,

C2=110×10318637.942.799

C2=1.916nF

C1=1.916×10-9×4.856-1

C1=7.388nF

R2=4.856 18637.947.388×10-9

R2=16.00kΩ

By putting the values in, the obtain circuit will be like this,